Operating Instructions and Parts Manual

16-inch Planer

Model JWP-16OS

WALTER MEIER (Manufacturing) Inc.

427 New Sanford Road

LaVergne, TN 37086	Part No. M-708531

Ph.: 800-274-6848	Revision B1  06/2013

www.waltermeier.com	Copyright © 2013 Walter Meier (Manufacturing) Inc.

1

Warranty and Service

Walter Meier (Manufacturing) Inc., warrants every product it sells. If one of our tools needs service or repair, one of our Authorized Service Centers located throughout the United States can give you quick service. In most cases, any of these Walter Meier Authorized Service Centers can authorize warranty repair, assist you in obtaining parts, or perform routine maintenance and major repair on your JET® tools. For the name of an Authorized Service Center in your area call 1-800-274-6848.

MORE INFORMATION

Walter Meier is consistently adding new products to the line. For complete, up-to-date product information, check with your local Walter Meier distributor, or visit waltermeier.com.

WARRANTY

JET products carry a limited warranty which varies in duration based upon the product (MW = Metalworking, WW = Woodworking).

WHAT IS COVERED?

This warranty covers any defects in workmanship or materials subject to the exceptions stated below. Cutting tools, abrasives and other consumables are excluded from warranty coverage.

WHO IS COVERED?

This warranty covers only the initial purchaser of the product.

WHAT IS THE PERIOD OF COVERAGE?

The general JET warranty lasts for the time period specified in the product literature of each product.

WHAT IS NOT COVERED?

Five Year Warranties do not cover woodworking (WW) products used for commercial, industrial or educational 
purposes. Woodworking products with Five Year Warranties that are used for commercial, industrial or education 
purposes revert to a One Year Warranty. This warranty does not cover defects due directly or indirectly to misuse, 
abuse, negligence or accidents, normal wear-and-tear, improper repair or alterations, or lack of maintenance.

HOW TO GET SERVICE

The product or part must be returned for examination, postage prepaid, to a location designated by us. For the name of the location nearest you, please call 1-800-274-6848.

You must provide proof of initial purchase date and an explanation of the complaint must accompany the merchandise. If our inspection discloses a defect, we will repair or replace the product, or refund the purchase price, at our option. We will return the repaired product or replacement at our expense unless it is determined by us that there is no defect, or that the defect resulted from causes not within the scope of our warranty in which case we will, at your direction, dispose of or return the product. In the event you choose to have the product returned, you will be responsible for the shipping and handling costs of the return.

HOW STATE LAW APPLIES

This warranty gives you specific legal rights; you may also have other rights which vary from state to state.

LIMITATIONS ON THIS WARRANTY

WALTER MEIER (MANUFACTURING) INC., LIMITS ALL IMPLIED WARRANTIES TO THE PERIOD OF THE LIMITED WARRANTY FOR EACH PRODUCT. EXCEPT AS STATED HEREIN, ANY IMPLIED WARRANTIES OR MERCHANTABILITY AND FITNESS ARE EXCLUDED. SOME STATES DO NOT ALLOW LIMITATIONS ON HOW LONG THE IMPLIED WARRANTY LASTS, SO THE ABOVE LIMITATION MAY NOT APPLY TO YOU.

WALTER MEIER SHALL IN NO EVENT BE LIABLE FOR DEATH, INJURIES TO PERSONS OR PROPERTY, OR 
FOR INCIDENTAL, CONTINGENT, SPECIAL, OR CONSEQUENTIAL DAMAGES ARISING FROM THE USE OF 
OUR PRODUCTS. SOME STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OF INCIDENTAL OR 
CONSEQUENTIAL DAMAGES, SO THE ABOVE LIMITATION OR EXCLUSION MAY NOT APPLY TO YOU.

Walter Meier sells through distributors only. The specifications in Walter Meier catalogs are given as general information and are not binding. Members of Walter Meier reserve the right to effect at any time, without prior notice, those alterations to parts, fittings, and accessory equipment which they may deem necessary for any reason whatsoever. JET® branded products are not sold in Canada by Walter Meier.

2

Table of Contents

Table of Contents	3

Warning	4

Safety Decals	5

Features of the JWP-16OS Planer	6

Specifications	6

Receiving	7

Parts Inventory	7

Assembly	8

Stand Assembly	8

Planer Unit Placement	8

Starter Box	9

Handwheel	9

Extension Rollers	9

Dust Collection Hood	9

Electrical Connections	10

Extension Cords	10

Adjustments	10

Overview	10

Control and Adjustments	10

Pulleys and Belts	11

Pulley Alignment	11

Belt Tension	11

Table Rollers	12

Extension Roller	12

Depth of Cut Adjustment	13

Knife Adjustment	13

Removing and Replacing Knives	15

Table Adjustment	15

Checking Work Table Parallel to Cutterhead	16

Adjusting Work Table Parallel to Cutterhead (Fine Adjustment)	16

Adjusting Work Table Parallel to Cutterhead (Major Adjustment)	16

Transmitting Rollers	17

Infeed & Outfeed Roller Spring Tension Adjustment	17

Anti-Kickback	18

Checking and Adjusting the Feed Roller Height	18

Gearbox	19

Operation	19

Table Locks	20

Power Feed	20

Handwheel	20

Depth Limiter	20

Initial Startup	21

Maintenance	21

General	21

Sharpening Knives	21

Lubrication	23

Optional Accessories	24

Troubleshooting	25

Performance Troubleshooting	25

Mechanical Troubleshooting	26

Parts List for the JWP-16OS Planer	27

Head Assembly	27

Head Assembly Parts List	28

Base Assembly	30

Base Assembly Parts List	30

Gear Box Assembly	32

Gear Box Assembly Parts List	33

Stand Assembly	34

Stand Assembly Parts List	34

Electrical Schematic for JWP-16OS Planer	35

3

W arning

1. 	Read the manual. Always read the owner’s manual carefully before attempting to use the machine.

Know the limitations and hazards associated with the use of this planer.

2. 	Installation. If mounting machine to the floor, use high quality anchor bolts through the mounting

holes on the base. If using a mobile base, be sure to lock the wheels.

3. 	Eye protection. Always wear approved safety goggles, glasses, or a face shield when operating this

machine. NOTE: Common eyeglasses are only impact resistant; they are not safety glasses. Also use face or dust mask if the cutting operation is dusty.

4. 	Dress code. Do not wear loose clothing, neckties, jewelry, or gloves that can get caught in moving

parts. Confine long hair. Keep sleeves above the elbow.

5. 	Placement. Place machine so that potential kickback area is not in line with aisles, doorways, wash

stations or other work areas. Do not use machine in a damp or wet location, or expose to rain. Keep work area well lighted.

6. 	Electrical grounding. Your machine must be electrically grounded. If a cord and plug are used,

make certain the grounding lug connects to a suitable ground. Follow the grounding procedure indicated by the National Electrical Code.

7. 	Guards. Be sure machine guards are in place and in good working order. Do not operate while gear

cover is open. If a guard must be removed for adjustments or maintenance, it should be reinstalled immediately upon completion of the procedure and before operating the machine.

8. 	Housekeeping. Before turning on machine, remove all extra equipment such as keys, wrenches,

scrap, stock, and cleaning rags from the machine. Keep the area around machine clean and free of scrap material and sawdust to minimize the danger of slipping.

9. 	Power off. Make sure the machine is either unplugged or electrically disconnected and locked out

when performing maintenance or service work. Also, make sure switch is in OFF position before plugging in power cord. Never leave the machine running unattended. Do not leave machine until it comes to a complete stop.

10. Cutterhead. Keep knives sharp and free of all rust and pitch. Make sure gib screws are tightened 
	securely.

11. Work piece. Check material for loose knots, nails and other defects that can damage knives and 
	pose a safety hazard for the operator.

12. Keep hands away from feed rollers and cutterhead while operating.

13. Use the proper extension cord. Make sure your extension cord is in good condition. When using an 
	extension cord, be sure to use one heavy enough to carry the current your product will draw. An 
	undersize cord will cause a drop in line voltage resulting in loss of power and overheating. For runs 
	up to 25 feet, use an 18 AWG or larger gauge cord.

14. Do not operate this machine while under the influence of drugs, alcohol or any medication.

15. If you are not thoroughly familiar with the operation of wood planers, obtain advice from your 
	supervisor, instructor or other qualified person.

16. Health hazards. 	Some dust created by power sanding, sawing, grinding, drilling and other

construction activities contains chemicals known to cause cancer, birth defects or other reproductive harm.  Some examples of these chemicals are:

•   Lead from lead-based paint.

•   Crystalline silica from bricks and cement and other masonry products.

•   Arsenic and chromium from chemically treated lumber.

Your risk from these exposures varies, depending on how often you do this type of work.  To reduce 
your exposure to these chemicals, work in a well-ventilated area, and work with approved safety 
equipment, such as those dust masks that are specifically designed to filter out microscopic particles.

4

Familiarize yourself with the following safety notices used in this manual.

This means that if precautions are not heeded, it may result in serious injury or

possibly even death.

This means that if precautions are not heeded, it may result in minor or moderate

injury and/or possible machine damage.

Safety Decals

Familiarize yourself with the location and content of these decals on your planer.

!

1.    Read instruction manual before operating machine.

2.    Do not operate without all guards properly installed.

3.    Remove or fasten loose articles of clothing such as neckties, etc.

Contain long hair.

4.    Remove jewelry such as finger rings, watches, bracelets, etc.

5.    Use approved safety glasses and/or face shield to protect eyes, and

use other personal safety equipment as required.  Do not wear 
gloves.

6.    Disconnect machine from power source before making any 
	adjustments or cleaning chips away from machine.

7.    Keep the floor around machine clean and free from scraps,

sawdust, oil and grease to minimize the danger of slipping.

8.    Do not operate this machine while under the influence of alcohol

or drugs.

9.    Failure to comply with these warnings may result in serious 
	personal injury.

DO NOT REMOVE OR OBSCURE THIS LABEL

5

Features of the JWP-16OS Planer

Specifications

Model Number	JWP-16OS

Stock Number		708531

Maximum Planing (W/in.)		16

Maximum Planing Thickness (T/in.)	6

Maximum Depth of Cut (in.)		3/16

Maximum Full Width Depth of Cut (in.)		1/8

Maximum Opening (in.)	6-3/8 x 16

Minimum Planing Length (in.)	8

Knives	3

Planer Blade Size (L x W x T/in.)	16-1/8 x 1 x 1/8

Cutterhead Speed (RPM)		4,500

Cuts per Minute		13,500

Cutterhead Diameter (in.)	2-7/8

Feed Rate (FPM)	16 & 20

Dust Port Diameter (in.)	4

Overall Dimensions (L x W x H/in.)	48 x 32 x 51

Footprint (LxW/in)	27-3/4 x 25-1/8

Motor	3HP, 1Ph, 230V

Net Weight (approx. lbs.)		430

The specifications in this manual are given as general information and are not binding. Walter Meier (Manufacturing) Inc., reserves the right to effect, at any time and without prior notice, alterations to parts, fittings, and accessory equipment deemed necessary for any reason whatsoever.

6

Receiving

Carefully unpack the planer and any loose items from the wood crate and inspect for damage. Any damage should be reported immediately to your  distributor  and  shipping  agent.  Before proceeding further, read your manual thoroughly to  familiarize  yourself  with  proper  assembly, maintenance and safety procedures.

Remove the protective coating from the table, bed rolls, feed rolls, cutterhead and loose items packed   with   the   machine,   including   lifting handles and motor pulley. This coating may be removed   with   a   soft  cloth  moistened   with Kerosene.  Do  not  use  acetone,  gasoline  or lacquer thinner for this purpose. Do not use solvents on plastic parts.

Parts Inventory

Parts requiring assembly:

1   Planer Unit

1	4" Dust Hood

2   In/Outfeed Extension Roller Assembly

1   Handwheel

Cast Foot Assembly:

4   Cast Feet

1   Hardware Bag, consisting of: 
	8   Socket Head Flat Screws

8   Flat Washers

8   Hex Nuts

Stand Assembly:

4   Stand Legs

2   Stand Braces - long

2   Stand Braces - short

2   Stand Top - front & rear

1   Stand Top - left

1   Stand Top - right

1   Hardware Bag, consisting of: 
	32  Carriage Bolts

32  Flat Washers

32  Hex Nuts

Accessory Package:

1   Handle

2   Knife Gauges

1   Knife Setting Gauge Bar

4   E-Rings for Knife Setting Gauge

4   Hex Wrenches - 3, 4, 5, 6mm

2   Open Wrenches - 8/10mm, 12/14mm

Hardware Bag for Handwheel:

1   M10 Hex Nut

1   M10 Flat Washer

1   Direction Label

Hardware Bag for Infeed/Outfeed Extension Rollers

6   M8 x 20 Hex Head Screws

6   M8 x 12 Set Screws

6   M8 Flat Washers

Hardware Bag for Dust Hood:

6   M6 x 12 Hex Head Screws

Hardware Bag for Base to Stand:

4   M8 x 30 Hex Head Screws

4   M8 Hex Nuts

8   M8 Flat Washers

7

Assembly

Most   of   the   JWP-16OS   Planer   has   been assembled at the factory. However some parts must be assembled after delivery.

Use care when cleaning the cutterhead; the knives are very sharp.

Stand Assembly

Refer to Figure 1.

1.   Cast Foot Assembly - Using parts from the 
	cast foot assembly hardware bag, mount

casting (2) to leg (1) by inserting two socket head flat screws (3) through the casting (2) and leg (1). Place a flat washer (4) and hex nut (5) on the screw and tighten.

Repeat for remaining three legs.

2.   Stand Top (front) - The front and rear top 
	stands (braces) are identical except for the

JET logo on the front piece.

Mount one end of the stand top (6) to a leg

(1) and fasten using two each carriage bolts

(7), flat washers (8) and hex nuts (9).

Note: Do not over tighten at this time. This will be the final step in the assembly.

3.   Fasten a second leg to the other end of the 
	stand top, repeating the step above.

4.   Take one long stand brace (10) and with 
	(4 ea) bolts, washers, and hex nuts fasten to

the legs of the assembly just completed in the steps 2 and 3. Fasten to the mounting holes in the middle of each leg.

5.   Set this assembly aside for now.

6.   Stand Top (rear) - Repeat steps 2-4 using 
	parts still remaining.

7.   Take the left stand top (11)

Note: The right stand top (13) has a cutout and will be used later.

Secure it to left side of the left legs (top mounting   holes)   of  the  front   and   rear assemblies previously constructed (the JET logo is the front) using 4 ea carriage bolts, flat washers and nuts.

8.   Take one short stand brace (12) and secure 
	to the left legs of the front and rear assembly

with (4 ea) carriage bolts, washers and nuts.

9.   Take the right stand top (13) - this piece has 
	the cutout - and secure it to the front and 
	rear legs (top mounting holes) on the right

side of the assembly using 4 ea carriage bolts, washers and nuts.

10. Secure the remaining short stand brace (14) 
	to the front and rear legs (middle mounting

holes) on the right side of the stand with (4 ea) carriage bolts, washers and nuts.

11. Make sure that the stand is symmetrical and 
	level.  Adjust  if  necessary  and  securely

tighten all bolts.

12. Before mounting the planer on the stand, 
	locate the stand on a solid, level foundation

to  ensure  best  planing  performance  and 
anchor to the floor with good quality lag 
screws.

Figure 1

Planer Unit Placement

There are four lifting handles (Fig.  2) on the machine.   The   handles   can   be   pulled   out (A, Fig. 2) for use when the planer is to be lifted and moved and slides into the body casting (B, Fig. 2) when not needed.

If a sling or forklift is used to lift the machine, be sure  to  lift  by  the  handles  only.  Make  sure machine is kept in level position while lifting.

Set   the   machine   on   the   stand   that   was 
assembled in the previous section, then secure 
the base to the stand using parts provided in the 
base to stand hardware bag, consisting of 4 ea 
M8 x 30 hex cap screws, 4 ea M8 hex nuts, and

8 ea M8 flat washers.

8

5.   Mount   the   handle	(C,   Fig.	4)   for   the

handwheel  in  the  threaded  hole  in  the 
handwheel,   and   tighten   with   a   wrench 
placed over the flat (12mm) on the handle.

Figure 2

Starter Box

The switch assembly (starter box and bracket) is

pre-assembled at the factory and simply needs 
to be mounted to the head casting on the left 
side of the machine with two socket head cap 
screws (A, Fig. 3) already in place the head 
casting.

Figure 3

Handwheel

1.   Remove the tape holding the key in the 
	shaft.

2.   Place the handwheel (B, Fig. 4) onto the 
	shaft;  making  sure  it  is  oriented  so  the

handwheel slips over the key.

3.   Remove  the  adhesive  backing  from  the 
	direction label supplied with the hardware

bag for the handwheel. Place the label onto 
the shaft and press it on the handwheel (D, 
Fig. 4).

4.   Place flat washer and hex nut on shaft and 
	tighten with wrench.

Figure 4

Extension Rollers

Mount two extension rollers to the table using the provided 6 ea M8 x 20 hex cap screws (12mm), 6 ea M8 flat washers, and 6 ea M8 x 12 set screws (A, Fig. 5). The rollers should be adjusted before operating the planer (this will be explained later in the Adjustment section).

Figure 5

Dust Collection Hood

Referring to Figure 6:

The dust collection hood (A) comes standard with the model JWP-16OS planer, and helps maintain  a  clean  and  safe  work  area.  It  is assembled to the planer (B) with six hex head screws (C) as shown.

9

Length of Cord

Rated 
Amps

25 ft	50ft	100ft

0-6	18	16	16

7-10	18	16	14

11-12	16	16	14

13-16	14	12	12

17-20	12	12	10

Figure 6

Electrical Connections

Electrical connections must be made by a

21-30	10	10

Figure 7

Adjustments

Overview

Not 
recommended

qualified electrician in compliance with all relevant   codes.   The   machine   must   be properly grounded to help prevent electrical shock and possible fatal injury.

A   power   plug   is   not   provided   with   the JWP-16OS planer. You may either connect a plug or "hard-wire" the machine directly to your electrical panel provided there is a disconnect near the machine. Consult the wiring diagrams at the end of this manual.

This  machine  must  be  grounded.  Grounding provides a path of least resistance to help divert current  away  from  the  operator  in  case  of electrical malfunction.

Make sure the voltage of your power supply matches the specifications on the motor plate of the machine.

Extension Cords

The   use   of   an   extension   cord   is   not 
recommended for this machine, but if one is 
necessary make sure the cord rating is suitable 
for the amperage listed on the machine's motor 
plate. An undersized cord will cause a drop in 
line  voltage  resulting  in  loss  of  power  and 
overheating.

The chart in Figure 7 shows the correct size cord to use based on cord length and motor plate  amp  rating.  If  in  doubt,  use  the  next heavier gauge. The smaller the gauge numbers the heavier the cord.

Many parts are factory adjusted. The operator should be familiar with the following procedures to gain a better understanding of the JWP-16OS Planer's construction and operation.

Control and Adjustments

Refer  to  Figure  8  for  general  control  and adjustment locations for the JWP-16OS Planer and are also listed below.

A - Switch

B - Handwheel 
C - Table Rollers

D - Feed Rate Change Knob

E - Two Table Lock Knobs on Opposing 
	Corners

Figure 8

10

Pulleys and Belts

The belt and pulley assembly are on the left side 
of the planer. To inspect for pulley alignment 
and correct belt tension, remove the four hex 
head screws (10mm) holding the cover in place. 
Figure 9 shows the assembly with the cover 
removed.

Pulley Alignment

1.   Place the edge of a metal ruler so it rests 
	perpendicular against the flat sides of the 
	motor and cutterhead pulleys (Fig. 11).

If the pulleys lie in straight plane (Fig. 11) they are aligned.

Figure 9

If the pulleys need to be aligned:

2.   Loosen  four  bolts  (12mm)  on  the  motor

mounting base plate (Fig.  10), which will

allow the motor to move from side-to-side.

Figure 10

3.   Adjust the motor until the pulleys are in the 
	aligned position as shown in Figure 11.

4.   Re-tighten all bolts.

Figure 11

Belt Tension

Check the belt tension by squeezing the belts 
together in the middle (between the motor and 
cutterhead pulleys as shown in Fig. 12) with 
moderate pressure. Proper tension is indicated 
when there is approximately 1/4” deflection.

Figure 12

To adjust the belt tension:

1.   Loosen the two 17mm bolts (A, Fig. 13) that 
	hold   the   motor/pulley   assembly   to   the

planer.

2.   Using a lever, raise the motor to increase 
	the belt tension.

Note: Belts will rarely be too tight. Adjustment typically requires belt tightening.

3.   Tighten the bolts and check the belt tension 
	again.

4.   Verify that the pulleys are still in alignment.

11

Figure 15

To adjust the rollers (refer to Figure 16):

1.   Disconnect machine from power source.

2.   Select a side (left or right) and lay a steel 
	ruler or straightedge across both rollers (A).

3.   On the side selected, loosen the screws (B) 
	with an Allen wrench, and turn the eccentric

Figure 13

Table Rollers

Two  table  rollers  (A,  Fig.  14)  ease  stock 
movement as it is fed through the planer. The 
height of the rollers is dependent on the type of 
wood being planed. When planing rough stock, 
set the rollers slightly high to keep the lumber 
from dragging along the table. Smooth lumber 
should be planed with the rollers set just above 
the plane of the table.

Figure 14

The two table rollers are preset at the factory at

0.004" above the table. The height adjustment range is from 0 to 0.06" (Fig. 15).

shafts (C) (12mm hex head adjustment) to raise or lower the rollers to the desired level. Use   a   feeler   gauge   to   measure   the clearance between the table and the bottom of the straightedge.

4.   When the proper height is achieved, tighten 
	screws (B).

Adjust the rollers from the opposite side of the table in the same manner.

Important:  Measure  in  several  places.  This measurement must  be  consistent  across  the entire table.

Figure 16

Extension Roller

Referring to Figure 17:

Place a straight edge (A) over the extension 
roller (B)  and  the  table  to  make  sure  the 
extension  roller  and  table  are  at  the  same 
height.

12

Note:  The  JWP-16OS  Planer  has  two  lock 
knobs;   one   is   located   by   the   handwheel 
(C, Fig. 19). The other is located on the opposite 
(left rear) corner.

Always   tighten   the   lock   knobs   before operating the planer.

Figure 17

Referring  to  Figure	18	-  if  adjustment  is

required:

1. 	Adjust the tightness of the mounting bolts

(A) and the depth of the setscrews (B) on the extension roller frame.

Various combinations of loosening and/or 
tightening of the setscrews and bolts will be 
required to level the extension rollers with 
the table. Adjustment is complete when the 
straightedge is level with the table and all 
three   rollers   touch   the   bottom   of   the 
straightedge.

2.	Adjust both front and rear extension rollers

in the same manner.

Figure 19

Knife Adjustment

When  checking  or  adjusting  the  cutterhead knives, proceed as follows:

1.   Disconnect machine from power source.

2.   Remove four hex cap screws (A, Fig. 20) 
	from the upper cover and three from the 
	bottom   of   the   dust   port.   Remove   the 
	cover/dust port (B, Fig 20) as a unit.

The cutterhead assembly can be seen from the opening on top.

Figure 18

Depth of Cut Adjustment

Refer to Figure 19.

The cutting depth scale (A) is a combination 
inch/metric scale with a cutting range from 0 to 
6” (150mm).   The   distance   of   upward   or 
downward   movement   is   controlled   by   the 
handwheel (B). One revolution of the handwheel 
is 0.158” (4mm).  Before  moving  the  head 
assembly up or down, loosen the lock knobs

(C,). After obtaining the proper height for the head assembly, tighten the lock knobs.

Motor assembly removed for clarity

Figure 20

13

Planer knives are dangerously sharp. Use 
extreme caution when inspecting, removing, 
sharpening,  or  replacing  knives  into  the 
cutterhead.  Failure  to  comply  may  cause 
serious injury.

3.   To check and adjust the knives, use the 
	provided knife gauge (Fig. 22) and check all 
	three knives. Press the knife gauge against 
	the cutterhead as shown in Figure 23.

Referring to Figure 21:

Knives (A) should just contact the bottom of 
the center protrusion (B) of the knife gauge

(F).

NOTE: If you are using a dial indicator  (not 
provided) to adjust the knives, the knives should 
protrude 0.059” (1.5mm) from the cutterhead.

Important:  Insure  that  the  feet  of  the  knife gauge sit flush against a clean surface of the cutterhead. Make sure that no dust, pitch, or lubrication buildup is present, which can cause errors in the knife blade settings.

4.   If an adjustment to one or more of the knives 
	is necessary, slightly loosen the knife gib (C)

by turning the six gib screws (D) into the gib. Turn  the  screws  just  enough  to  relieve stress in the cutterhead without disturbing the setting of the knives. Do this for all three knives at the same time.

5.   With the gauge (F) in place over a knife (A) 
	continue to loosen the gib screws (D) until

the springs (E) begin raising the knife.

When  knife  comes  into  contact  with  the 
center protrusion (B) of the gauge, adjust 
the jack screws higher or lower to touch the 
bottom of the knife, then snug up the gib by 
lightly backing out the six gib screws (D) 
against the slot.

Note: At this time, only tighten the knife in the slot just enough to hold knife in position.

Figure 21

6.   If additional knives must be reset, repeat 
	steps 4 and 5 for each knife.

7.   After all three knives are set with screws just 
	snug,  back  out  and  tighten  the  six  gib

screws (D, Fig. 21 & Fig. 23) of the first knife 
against the slot starting with the end screws, 
then the center screws, until the knife is 
securely  held  in  the  cutterhead.  Tighten 
remaining two knives in the same manner.

Important:  Double  check  all  gib  screws  for tightness.

Figure 22

14

7.   Reinsert springs, knives and gib into slot of 
	the cutterhead. Back out gib screws just 
	enough to hold the knife in the cutterhead.

8.   Place knife gauge (Fig. 23) over knife. Still referring to Figure 21:

9.   While  holding  down  on  the  knife  gauge, 
	loosen all six gib screws (D) by turning them 
	into the gib (C) until cutting edge of knife 
	comes into contact with the protrusion of the 
	gauge (B). Adjust the jack screws higher or 
	lower to touch the bottom of the knife. Snug 
	up the gib by slightly backing out the six 
	locking screws against the slot.

Figure 23

Removing and Replacing Knives

Planer knives are dangerously sharp. Use extreme caution when inspecting, removing, sharpening,  or  replacing  knives  into  the cutterhead.  Failure  to  comply  may  cause serious injury.

To remove a knife:

1.   Disconnect machine from power source.

2.   Remove four screws (A, Fig. 20) from the 
	upper cover and three from the bottom of

the dust port. Remove the cover/dust port (Fig. 20) as a unit.

The cutterhead assembly can be seen from the opening on top.

Refer to Figure 21:

3.   Loosen the gib (C) by turning the six gib 
	screws (D) into the gib. Remove gib (C),

knife (A) and springs (E).

Note: The inner two springs may pop out when the knife and gib are removed.

4.   Remove the remaining two knives in the 
	same manner.

5.   Thoroughly   clean   the   knife   slots,   gibs, 
	springs  and  gib  screws.  Check  the  gib

screws;  if  the  threads  appear  worn  or stripped  or  if  the  heads  are  becoming rounded, replace them.

6.   Inspect the cutting edge of the knives for 
	nicks or wire edge. It is recommended that

knives be replaced when they become dull 
or damaged. If they are to be reused, refer 
to Sharpening Knives in the Maintenance 
Section.

Note: At this time only, tighten the knife into 
the slot just enough to hold the knife in 
position.

10. Replace and reset the other two knives by 
	repeating steps 3 - 9.

11. After all three knives are set with the screws 
	just  snug,  back  out  and  tighten  the  six

screws (D) against the slot starting with the end screws first and then the center screws until  the  knife  is  securely  held  in  the cutterhead.   Tighten   the   remaining   two knives in the same manner.

Important:  After replacing and checking knives, CHECK  AGAIN  carefully.  Make  certain  the direction of knives is correct and all eighteen (3 knives   x 6   screws)   locking   screws   are tightened securely.

Table Adjustment

To perform the table adjustments described in the next two sections, you will need a straight edge, feeler gauge, and a home made gauge block made of hardwood. Make the gauge block by following the dimensions shown in Fig. 24. Precision adjustments require accuracy when milling the gauge block.

Figure 24

15

Checking Work Table Parallel to Cutterhead

The worktable is set parallel to the cutterhead at 
the factory and no further adjustment should be 
needed. If your machine is planing a taper, first 
check to see if the knives are set properly in the 
cutterhead. Then check to see if the worktable is 
set parallel to the cutterhead by proceeding as 
follows:

1.   Disconnect machine from power source.

2.   Turn the handwheel clockwise; raising the 
	cutterhead assembly high enough to place

the gauge block (A, Fig. 25) on the work table under one end of the head casting (B, Fig. 25).

3.   Turn  the  handwheel  to  lower  the  head 
	assembly until the head casting body barely

touches the gauge block. The blades should not touch the block.

4.   Slide the block toward the opposite side of 
	the head casting. Use a feeler gauge to

measure  the  width  of  the  gap,  if  any, 
between the top of the block and the bottom 
of the cutterhead. Make a note of the gap, if 
any.

5.   If the block wedges tightly between the table 
	and the head casting when shifting from one

side to the other, repeat steps 2 through 4, 
but start from the opposite end of the head 
casting.

Figure 25

If the gap difference from one side to the other is equal   to   or   less   than 0.004",   no   further adjustment is necessary

If the gap difference from one side to the other side is grater than 0.004", but less than 0.016", go  to  the  Adjusting  Work  Table  Parallel  to Cutterhead (Fine Adjustment) section.

It the gap difference from one side to the other is 
grater  than 0.016",  the  cutterhead  assembly 
raising chain under the planer base needs to be

adjusted - see the Adjusting Work Table Parallel to Cutterhead (Major Adjustment) section.

Adjusting Work Table Parallel to Cutterhead (Fine Adjustment)

If the gap difference determined in the previous section is greater than 0.004" and less than

0.016", perform the adjustment procedure as follows:

1.   Determine which side of the table must be 
	raised to correct the gap.

2.   Locate the two socket head cap screws in 
	the table casting for each of the columns (A,

Fig.  26). Loosen both sets of screws for 
each column on the side you wish to adjust.

3.   Push   down   or   pull   up   the   cutterhead 
	assembly in the desired direction. Hold the

assembly in position and retighten the cap screws.

4.   Recheck the table to cutterhead parallelism 
	again as described in the previous section,

then repeat steps 1 - 3 until the deviation is less than 0.004".

Figure 26

Adjusting Work Table Parallel to 
Cutterhead (Major Adjustment)

Refer to Figure 27.

1.   Disconnect the machine from power source.

2.   On the underside of the base, remove bolt 
	(A) and loosen bolt (B) which will allow you

to move the idler sprocket assembly (C) far enough to release tension on the chain.

3.   Remove chain from the particular sprocket 
	on the corner of the base that must be

adjusted.

16

4.   Turn the sprocket by hand to bring that 
	corner into adjustment with the other three 
	corners.

Note: Turning sprocket clockwise will increase 
the  distance  between  the  working  table  and 
head casting; counter-clockwise will decrease 
the distance. This adjustment is very sensitive -
one revolution of the lead screw equals 0.158” 
(4mm) of travel. It should not be necessary to 
turn the sprocket more than one or two teeth.

5.   When   adjustments   are   correct,   replace 
	chain  around  the  corner  sprocket,  slide

sprocket	(C)   back   to   re-tension   chain,

tighten  bolt  (B)  and  replace  and  tighten 
bolt (A).

Note: It may be necessary to perform the Fine Adjustment procedure after the major to achieve the best result.

Figure 27

Transmitting Rollers

Figure 28 show the positions of the rollers that are listed below and described in the following sections.

A   Anti Kickback Fingers B   Infeed Roller

C   Chipbreaker 
D   Cutterhead

F   Outfeed Roller

Figure 28

Infeed & Outfeed Roller Spring Tension Adjustment

The infeed roller (B, Fig. 28) and outfeed roller (F, Fig. 28) feed the stock while it is being planed. The infeed and outfeed rollers are under spring tension and this tension must be sufficient to feed the stock uniformly through the planer without slipping but should not be so tight that it causes damage to the board and/or the rubber coating on the outfeed roller. The tension should be equal at both ends of each roller.

To adjust the spring tension of the infeed and 
outfeed rollers, turn screws (A, Fig. 29) with a 
hex wrench. A clockwise turn increases tension 
on    the    pressure    spring 	(Fig. 	30);    a 
counterclockwise turn decreases tension. Adjust 
the screws at the other end of the rollers with the 
same number of turns.

Note: The most effective pressure settings are 
dependent on the type of lumber being planed. 
Experimentation will determine the best settings.

Figure 29

17

Anti-Kickback

The   JWP-16OS   Planer   provides   an   anti-
kickback   safety   feature.   The   anti-kickback 
fingers hang from a rod suspended across the 
front of the cutterhead casting (A, Fig. 28) and 
help prevent kickback of stock. It is necessary to 
inspect them regularly to make sure they hang 
freely. Check that they are free of gum and pitch 
to insure independent movement and correct 
operation.

Figure 30

Figure 31

Checking and Adjusting the Feed Roller Height

The infeed and outfeed rollers propel the lumber through the planer. The rollers also press the lumber flat against the planer table.

The infeed and outfeed rollers are adjusted at the factory and are set at 0.020" below the knife-
edge at bottom dead center (Fig. 31).

Note: Before proceeding with this adjustment, make sure the knives are adjusted properly as outlined in the Knife Adjustment section.

Remove the drive chain cover to access the 
roller adjustments on the drive chain side of the 
planer.

To check or verify the roller height:

1.   Disconnect  the  machine  from  the  power 
	source.

2.   Turn the handwheel clockwise, raising the 
	cutterhead assembly high enough to place

the gauge block (J, Fig. 32) on the table 
directly 	underneath 	the 	cutterhead 
(D, Fig. 32) Using a 0.02” (0.5mm) feeler 
gauge (K, Fig. 32) placed on top of the 
gauge block, lower the head assembly until 
the knife just touches the feeler gauge when 
the knife is at its lowest point. Do not move 
the working table any further until the infeed 
and outfeed rollers are adjusted.

18

The gearbox is located on the right side of the 
planer directly behind the table lock knob (A, 
Fig. 35). The gearbox transfers power from the 
belt-driven cutterhead to the power feed rollers. 
A   two-speed   transmission,   controlled   by   a 
push/pull lever (D), moves the stock through the 
planer at either 16 or 20 feet-per minute. The 
push/pull  lever  is  set  while  the  machine  is 
running.

Figure 32

3.   Move the gauge block (J, Fig. 33) under one 
	end of the outfeed roller (F, Fig. 33). The 
	bottom of the outfeed roller should just touch 
	the top of the gauge block. If an adjustment 
	to the outfeed roller is necessary, loosen the

jam   nut	(L,   Fig	33)   and   turn   screw

(M, Fig. 33)  until  the  outfeed  roller  just 
touches the gauge block. Then tighten jam

nut (L, Fig. 33).

4.   Slide the gauge block to the other end of the 
	outfeed  roller  and  repeat  the  adjustment

described in Step 3.

5.   Recheck the settings and repeat steps 1-4 if 
	necessary.

Figure 33

Gearbox

Referring to Figure 34:

The JWP-16OS Planer is equipped with a spiral, 
serrated infeed roller (F) and a solid outfeed 
roller. When the feed rollers are engaged, they 
turn to feed the stock. The feed rollers are driven 
by chains (A) and sprockets (B) in the gearbox.

19

Figure 34

Operation

The    JWP-16OS    Planer    is    a    powerful 
woodworking 	machine, 	designed 	and 
constructed for professional-quality applications.

Because of its powerful motor and razor-
sharp   knives,   the   JWP-16OS   Planer   is 
inherently    dangerous    and    should    be 
operated   with   considerable   caution   and 
respect.  Failure  to  do  so  could  result  in 
damage to the machine or severe injury to 
the operator or others in the work area.

Overview

There are a number of safety issues that relate directly to the operation of the planer. Keep in mind  that  these  are  not  all-inclusive.  Work situations, wood types, and other variables that differ from shop to shop must be considered in order  to  operate  this  planer  safely.  Always consider safety and common sense first when operating this or other machinery.

1.   Always inspect lumber for defects (warping, 
	cupping, twisting, etc.). Do not use lumber of

questionable quality.

2.   Check lumber for nails, staples, imbedded 
	gravel, etc. before planing.

3.   Use the full width of the planer. Alternate 
	between the left, the right and the middle

when feeding lumber into the planer. Your knives will remain sharp much longer.

4.   Remove glues, epoxies and other foreign 
	matter before planing lumber.

5.   Never    attempt    to    plane    laminates, 
	particleboards, plastics or other man-made

materials.

6.   Plane  wood  with  the  grain.  Never  plane 
	across the grain.

7.   Do not use boards with loose knots, splits, 
	cross  grain  or  other  defects.  They  can

damage the machine and cause injury.

8.   Keep your work area clear.

9.   Wood with excessive water content or wood 
	exposed  to  rain, ice,  or  snow  will  plane

poorly and cause excess wear to the knives and  motor.  Excessive  moisture  will  also hasten rust and corrosion.

10. Learn as much as possible about planing 
	procedures. Alternative publications present

more wood specific planing requirements.

Table Locks

Before attempting to adjust table height, loosen the two table lock knobs, one on the right front of the head casting assembly (A, Fig. 35) and the other on the left rear.

Figure 35

After  the  head  assembly  height  is  adjusted, tighten the two black knobs down again.

Power Feed

Referring to Figure 36:

The power feed features two feed rates, 16 FPM 
and 20 FPM. When running the machine, the

20

operator can control the feed speed by moving 
the feed control knob. Moving the knob toward 
the machine produces the 20 FPM feed speed

(A), away from the machine produces 16 FPM

(C) and a center position places the gear box in neutral (B).

Figure 36

Set the feed rate while the planer is running but before feeding lumber into it. DO NOT change speeds after the cutting operation has begun.

If the cut is too large, the planer will slow 
down  considerably, possibly  even  stalling 
the motor. Turn off the power immediately, 
raise  the  head  assembly  and  remove  the 
workpiece. Re-adjust the head assembly to 
allow a lesser cut and repeat the operation.

Handwheel

Crank the handwheel to raise or lower the table according to the desired workpiece thickness.

Each  complete  revolution  of  the  handwheel 
moves the head assembly by 0.158” (4mm). 
Make sure the height scale is properly adjusted.

With the depth-limiting clip installed, you cannot cut full width more than 1/8" in a single pass (3/16" if lumber can fit on either side of the depth limiting clip). While cutting this much material is possible, it is not recommended.

Depth Limiter

The Model JWP-16OS Planer is equipped with a depth limiter - located on the bottom of the cutterhead casting just below the warning label (A, Fig. 37). The depth limiter controls maximum depth of cut to 1/8".

Close-fitting parts, such as the cutterhead slot and gibs, should be cleaned with a cloth or brush and non-flammable solvent, and free from clinging foreign matter.

Remove  resin  and  other  accumulations  from feed rollers and table with a soft rag and nonflammable solvent.

Periodically  check  all  the  chains  for  proper

Figure 37

To avoid mechanical damage to the planer, do not remove the depth limiter.

Initial Startup

After   the   assembly   and   adjustments   are complete the planer is ready to be tested.

Turn on the power supply at the main panel. 
Press the Start button. Keep your finger on the 
Stop button in case of a problem. The planer 
should run smoothly with little or no vibration or 
rubbing  noises.  Investigate  and  correct  the 
source of any problems before further operation.

DO NOT attempt to investigate or adjust the 
planer  while  it  is  running.  Wait  until  the 
planer  is  turned  off,  unplugged  and  all 
working  parts  have  come  to  a  complete 
standstill.

Always wear ANSI-approved safety glasses or goggles when operating equipment.

Maintenance

General

Inspect  your  planer  each  time  before  using. Check for the following conditions and repair or replace when necessary.

1.   Loose mounting bolts.

2.   Worn switch.

3.   Worn or damaged cords and plugs.

4.   Damaged V-belts.

5.   Any other condition that could hamper the 
	safe operation of this machine.

Buildup of sawdust and other debris can cause your  machine  to  plane  inaccurately.  Periodic cleaning  is  not  only  recommended  but  also mandatory for accurate planing.

tension and adjust accordingly if required.

Tip: If a foreign object nicks the knives, instead of throwing them away or trying to grind out the deep nick, simply stagger the knives in the head, moving one knife no more than 1/8" to the right and another knife no more than 1/8" to the left. The nick should no longer be noticeable.

The table should be kept clean and free of rust. Some users prefer a paste wax on exposed steel and cast iron surfaces. The wax provides a layer of protection as well as reducing friction between lumber and the table making cuts faster and  smoother.  Avoid  any  wax  that  contains silicone or other synthetic ingredients. These materials can find their way into lumber and can make staining and finishing difficult.

Another option is talcum powder applied with a blackboard eraser rubbed in vigorously once a week; this  will fill casting pores and form a moisture   barrier.   This   method   provides   a tabletop that is slick and allows rust rings to be easily wiped from the surface. Another important fact is that talcum powder will not stain wood or mar finishes as wax pickup does.

Sharpening Knives

It is recommended that knives be replaced they become dull or damaged.

If knives are to be reused, it is recommended 
that they be sharpened by a professional knife 
sharpener.  Improperly  sharpened  knives  can 
cause a number of defects to lumber and put 
unnecessary  load  on  the  motor  and  drive 
systems.

If you must sharpen the knives yourself, please take note of the following information:

Primary Grinding Angle - 40 degrees 
Secondary Grinding Angle - 42 degrees

21

The grinding angle has been determined by the 
factory to be the best compromise for planing a 
wide variety of wood types. In most cases, that 
angle  will  produce  excellent  results.  If  you 
choose to change the angle of your bevel, be 
sure to consult with a trained sharpener or with a 
reference book before you commit to changing 
the angle of bevel.

Figure 38

22

Lubrication

The  bearings  on  the  cutterhead  are  factory 
lubricated and sealed for life - no lubrication 
required.

Gearbox - The oil in the gearbox must be drained and replaced after the first 20 hours of operation. (See the Lubrication Table).

Inspect levels periodically and change yearly. Replace gear oil more frequently under heavy use. Fill until the oil reaches the top of the filler plug port for correct oil level.

Figure 39

To replace the gearbox lubricant:

1.   Remove the drain plug (3. Fig. 39), and filler 
	cap (A, Fig. 40). Drain dirty oil thoroughly.

2.   Tighten the drain plug (3, Fig. 39).

3.   Fill   with   clean   lubricant   through   hole 
	(A, Fig. 40).

Tighten filler cap (A, Fig. 40).

Figure 40

No	Fig	Position	Interval	Suitable Types of Lubricant

1	39	Drive Chain	Inspect and lubricate monthly	Grease

2	39	Gear Box

When   operated   more   than 2,500 hours

HD-100, Mobil Gear 627, Shell Omala 100, ESSO Spartan EP-100

3	41	Lead Screw	Inspect and lubricate Monthly	Grease

4	41	Column	Inspect and lubricate weekly	SAE-30

5	42	Table Chain

6	43	Feed Rollers

Inspect   monthly	-   lubricate when needed

Inspect   and   lubricate   daily before startup

Grease or good quality bicycle chain lubricant

SAE-30

Lubrication Table

23

Figure 43

Figure 41

Figure 42

Optional Accessories

708814	Knives (set of 3)

708119		Universal Mobil Base

24

Troubleshooting

Performance Troubleshooting

Problem	Possible Cause	Solution

Snipe	Table rollers not set properly.	Adjust rollers to proper height

Inadequate support of long 
Note: Snipe can 	boards.

be minimized but

not eliminated	Uneven feed roller pressure front

to back.

Support long boards with extension rollers.

Adjust feed roller tension

Dull knives.	Sharpen knives.

Lumber not butted properly.	Butt end to end each piece of stock as they

pass through.

Fuzzy Grain	Planing wood with a high	Remove high moisture content from wood by

moisture content.	drying.

Dull knives.	Sharpen or replace.

Torn Grain	Too heavy a cut.	Adjust proper depth of cut

Knives cutting against grain.	Cut along the grain.

Dull knives.	Sharpen knives.

Rough/Raised	Dull knives.	Sharpen knives.

Grain

Too heavy a cut.	Adjust proper depth.

Moisture content too high.	Remove high moisture content from wood by

drying.

Rounded, glossy	Dull knives.	Sharpen or replace knives.

surface

Feed speed too slow.	Increase speed.

Cutting depth too shallow.	Increase depth.

Poor feeding of	Inadequate feed roller pressure.	Adjust feed roller tension. If proper tension

lumber.	cannot be achieve, replace feed rollers

Planer bed rough or dirty.	Clean pitch and residue, and wax planer table.

Transmission v-belt slipping.	Tighten transmission v-belt.

Surface of feed rollers clogged.	Clear pitch and residue out of teeth.

Uneven depth of	Knife projection.	Adjust knife projection.

cut side to side.

Cutterhead not level with bed.	Level bed.

Board thickness	Depth of cut scale incorrect.	Adjust depth of cut scale.

does not match 
depth of cut 
scale.

25

Mechanical Troubleshooting

Problem	Possible Cause	Solution

Chain jumping.	Inadequate tension.	Adjust chain tension.

Sprockets misaligned.	Align sprockets

Sprockets worn.	Replace sprockets.

Machine will	No incoming power.	Verify unit is connected to power

not start/ restart

or repeatedly	Overload automatic	When planer overloads on the circuit breaker built into

trips circuit	reset has not reset	the motor starter, it takes time for the machine to cool

breaker or	down before restart. Allow unit to adequately cool before

blows fuses.	attempting restart. If problem persists, check amp setting

on the motor starter inside the electrical box.

Planer frequently trips.

One cause of overloading trips, which are not electrical in 
nature, is too heavy a cut. The solution is to take a lighter 
cut. If too deep a cut is not the problem, then check the 
amp setting on the overload relay. Match the full load

amps on the motor as noted on the motor plate. If the 
amp setting is correct then there is probably a loose 
electrical lead. Check amp setting on motor starter.

Building circuit breaker	Verify that planer is on a circuit of correct size. If circuit

trips or fuse blows.	size is correct, there is probably a loose electrical lead.

Check amp setting on motor starter.

Loose electrical	Go through all the electrical on the planer including motor

connections.	connections, verifying the tightness of each. Look for any

signs of electrical arcing which is a sure indicator of loose connections or circuit overload.

Motor starter failure. 	Examine motor starter for burned or failed components. If

damage is found, replace motor starter. If motor starter

looks okay but is still suspect, you have two options: have a qualified electrician test the motor starter for function, or purchase a new starter and establish if that was the 
problem on changeout

Motor starter failure. 	If you have access to a voltmeter, you can separate a

starter failure from a motor failure by first, verifying

incoming voltage at 220+/-20 and second, checking the 
voltage between starter and motor at 220+/-20. If 
incoming voltage is incorrect, you have a power supply 
problem. If voltage between starter and motor is 
incorrect, you have a starter problem. If voltage between 
starter and motor is correct, you have a motor problem.

Motor failure.	If electric motor is suspect, you have two options: Have a

qualified electrician test the motor for function or remove the motor and take it to a quality electric motor repair shop and have it tested.

Miswiring of the unit. 	Double check to confirm all electrical connections are

correct and properly tight. The electrical connections

other than the motor are pre-assembled and tested at the 
factory. Therefore, the motor connections should be 
double checked as the highest probability for error. If 
problems persist, double-check the factory wiring.

26

Parts List for the JWP-16OS Planer

Head Assembly

27

Head Assembly Parts List

Index No.	Part No.	Description	Size	Quantity

1	JWP16OS-101	Pulley Guard	1

2	JWP16OS-102	V-belt	M27	3

3	TS-1491041	Hex Cap Screw	M10×30L	3

4	TS-1550071	Flat Washer	M10	4

5	JWP16OS-105	Pulley Cover	1

6	JWP16OS-106	Motor Cable	1

7	JWP16OS-107	Strain Relief	1

8	JWP16OS-108	Motor	1

9	JWP16OS-109	Hex Cap Screw	M10×80L	1

10	JWP16OS-110	Hex Flange Screw	M6×12L		27

11	JWP16OS-111	Spacer	1

12	TS-1490041	Hex Cap Screw	M8×25L	1

13	TS-1550061	Flat Washer	M8	9

14	JWP15H-014	Machine Pulley	1

15	TS-1540061	Hex Nut	M8		12

17	JWP16OS-117	Motor Mount	1

18	TS-1490081	Hex Cap Screw	M8×45L	4

19	PA-C61	Spring	1

20	PA-C58	Bracket	1

21	PA-C59	Shaft	1

22	PA-C57	Idle Pulley	1

23	PA-C56	Shaft	1

24	PA-C60	Hanger	1

25	TS-1503031	Socket Head Cap Screw	M6×12L	3

26	JWP16OS-126	Collector Hood	1

27	JWP16OS-127	Upper Cover	1

28	JWP15H-040	Deflector Plate	1

29	JWP16OS-129	Cap	3

30	TS-1504101	Socket Head Cap Screw	M8×50L	4

31	TS-1540072	Hex Nut	M10-1.25M	4

33	JWP16OS-133	Label, Direction	1

34	JWP16OS-134	Handwheel	1

35	JWP15H-060	Handle	1

36	JWP15H-109	Knob	1

37	JWP15H-108	Lock Bushing	2

38	TS-1482021	Hex Cap Screw	M6×12L	4

39	TS-1550041	Flat Washer	M6	6

40	JWP16OS-140	Head Casting	1

41	JWP15H-036	Plate Spring	3

42	JWP15H-021	Socket Set Screw	M22-1.5P×20L	4

43	TS-1523051	Socket Set Screw	M6×16L	4

44	JWP16OS-144	Shaft	2

45	JWP15H-106	Lock Bushing	2

46	PF-C22	Plate	2

47	JWP15H-062	Spring Pin	2

48	JWP15H-061	Cover	1

49	TS-1504091	Socket Head Cap Screw	M8×45L	1

50	JWP16OS-150	Ball Bearing	6205-2NSE	1

51	JWP16OS-151	Key	8×8×40	1

52	JWP16OS-152	Knife Bar	3

53	708814	Knives (Set of 3)	1

54	JWP16OS-154	Spring	6

55	TS-1513021	Socket Head Flat Screw	M5×12L	6

56	JWP16OS-156	Cutterhead	1

28

Head Assembly Parts List

Index No.	Part No.	Description	Size	Quantity

57	JWP15H-007	Screw		18

58	TS-1503041	Socket Head Cap Screw	M6×16L	8

59	TS-1524031	Socket Set Screw	M8×12L	1

60	JWP15H-020	Spring	4

61	JWP208-019	Bushing	4

62	JWP15H-022	Plate	4

63	TS-1490031	Hex Cap Screw	M8×20L	5

64	TS-1540041	Hex Nut	M6	6

65	JWP16OS-165	Outfeed Roller	1

66	JWP15H-046	Limiter Plate	1

67	TS-1534041	Machine Screw, Flat Head Phillips	M5-10L	2

68	JWP15H-026	Key	5×5×22	2

69	JWP16OS-169	Rivet	2

70	JWP15H-031	Sprocket	4

72	TS-1482031	Hex Cap Screw	M6×16L	2

73	JWP15H-073	Chain	06B×63P	1

74	JWP16OS-174	Chip Bracket	1

75	JWP16OS-175	Shaft	1

76	JWP16OS-176	Knob	1

77	JWP15H-045	Retaining Ring	ETW-15	2

78	JWP15H-042	Collar		45

79	JWP15H-041	Anti-Kick Finger		44

80	JWP16OS-180	Shaft	1

81	JWP16OS-181	Infeed Roller	1

82	JWP15H-027	Sprocket	1

83	JWP16OS-183	Power Cord	1

84	JWP16OS-184	Key	5×5×30	1

85	JWP15H-017	Motor Pulley	1

87	JWP16OS-187A	Switch	1

88	JWP15H-076A	Switch Plate	1

89	JWP16OS-189	Tooth Washer	2

90	JWP16OS-190	Screw	3/16-24 x 1-3/4	2

91	JWP16OS-191	Hex Nut	3/16-24	2

92	JWP16OS-192	Strain Relief	2

29

Base Assembly

Base Assembly Parts List

Index No. Part No.	Description	Size	Quantity

1	JWP16OS-201	Lifting Handle	4

2	JWP15H-207	Nut Fixture	4

3	JWP16OS-203	Leadscrew	3

4	JWP16OS-204	Column	3

5	TS-1523041	Socket Set Screw	M6×12L	4

6	JWP15H-203	Column	4

7	JWP16OS-207	Base	1

8	TS-1525021	Socket Set Screw	M10×12L	8

9	JWP16OS-209	Roller	2

10	BB-608Z	Ball Bearing	4

11	JWP15H-104	Eccentric Shaft	4

12	JWP15H-221	Bracket	1

13	JWP15H-222	Shaft	1

14	JWP15H-223	Sprocket	1

15	JWP15H-224	Retaining Ring	STW-15	1

16	JWP16OS-216	Chain	41×148P	1

17	TS-1550061	Flat Washer	M8		18

30

Base Assembly Parts List

Index No. Part No.	Description	Size	Quantity

18	TS-1490041	Hex Cap Screw	M8×25L	2

19	BB-6202ZZ	Ball Bearing	6202ZZ	4

20	JWP15H-214	Retaining Ring	RTW-35	4

21	JWP15H-216	Sprocket	4

22	TS-0680041	Flat Washer	3/8	4

23	TS-1540072	Nut	M10-1.25P	4

24	JWP15H-209	Retaining Ring	RTW-38	1

25	JWP15H-208	Bushing	1

26	JWP16OS-226	Key	4×4×20	1

27	JWP16OS-227	Leadscrew	1

28	JWP16OS-228	Scale	1

29	TS-1531012	Machine Screw, Pan Head Phillips	M3×6L	2

30	JWP16OS-230	Column	1

31	TS-1490031	Hex Cap Screw	M8×20L	6

32	TS-1524031	Socket Set Screw	M8×12L	6

33	TS-1490051	Hex Cap Screw	M8×30L	4

35	JWP16OS-235	E-ring	ETW-10	4

36	JWP15H-111	Roller Frame	2

37	TS-1482021	Hex Cap Screw	M6×12L		12

38	TS-1550041	Flat Washer	M6		12

39	JWP15H-113	Roller Liner		12

40	JWP15H-114	Roller Shaft	6

41	JWP15H-112	Roller	6

31

Gear Box Assembly

32

Gear Box Assembly Parts List

Index No.	Part No.	Description	Size	Quantity

1	TS-1550041	Flat Washer	M6	2

2	TS-1503031	Socket Head Cap Screw	M6×12L	1

3	BB-6204ZZ	Ball Bearing	6204ZZ	1

4	JWP15H-304	Gear	1

5	TS-1503061	Cap Screw	M6x25L	6

6	BB-6201	Ball Bearing	6201	1

7	JWP15H-309	Gear	47T	1

8	JWP15H-310	Gear	18T	1

9	JWP15H-313	Key	5x5x10	2

10	JWP15H-337	Cover	1

11	JWP15H-335	Pin	2

12	JWP15H-339	Oil Plug	2

13	OS-28408	Oil Seal	TC28x40x8	1

14	JWP15H-301	Gear Box	1

15	JWP15H-336	Gasket	1

16	BB-6201Z	Ball Bearing	6201Z	3

17	JWP15H-314	Shaft	1

18	JWP15H-320	Key	6x6x40	1

19	JWP15H-312	Gear	71T	1

20	JWP15H-334	Knob	1

21	JWP15H-325	Sprocket	12T	1

22	TS-1482031	Hex Cap Screw	M6x16L	1

23	JWP15H-318	Gear Assembly	96/92T	1

24	JWP15H-323	Shaft	1

25	JWP15H-322	Spring	1

26	JWP15H-321	Ball	1

27	OS-25476	Oil Seal	1

28	BB-6204Z	Ball Bearing	6204Z	1

29	JWP16OS-329	Hex Flange Screw	M6x12L	1

30	JWP15H-329	Clutch	1

31	JWP15H-330	Handle	1

32	JWP15H-333	Oil Seal	1

33	JWP15H-327	Chain	1

33

Stand Assembly

Stand Assembly Parts List

Index No. Part No.	Description	Size	Quantity

1	JWP16OS-401	Upper Side Brace (L)	1

2	JWP16OS-402	Upper Front Brace	2

3	JWP16OS-403	Down Side Brace (R)	2

4	JWP16OS-404	Down Front Brace	2

5	TS-1550061	Flat Washer	M8		48

6	TS-0561021	Hex Nut	5/16”-18		32

7	JWP16OS-407	Carriage Bolt	5/16”-18×14L		32

8	TS-1540061	Hex Nut	M8		12

9	TS-1515021	Socket Head Flat Screw	M8x20L	8

10	JWP16OS-410	Foot	4

12	JWP16OS-412	Leg	4

13	JWP16OS-413	Upper Side Brace (R)	1

34

Electrical Schematic for JWP-16OS Planer

A1	A

1L1	3L2	5L3	13NO

2T1	4T2	6T3	14NO

A2

R	R

O

ManAuto

98	97	96	95

17

35

WALTER MEIER (Manufacturing) Inc.

427 New Sanford Road

LaVergne, TN 37086 
Phone: 800-274-6848

www.waltermeier.com

36